# Chapter 165:			GENERAL PERMIT FOR CLASS IV-A INCINERATORS

SUMMARY: This Chapter regulates air emissions from Class IV-A Veterinary Incinerators and Human Crematory Incinerators. Owner/operators of Class IV-A Incinerators may obtain specific regulatory coverage under this General Permit regulation in lieu of an individual air emission license if their facility qualifies as a minor source of air emissions. Such parties shall do so by obtaining a Class IV-A Incinerator General Permit (GP) for each Class IV-A Incinerator unit by submitting a General Permit application with Notification of Intent to Comply (NOITC) which will attest to their formal agreement to abide by all conditions contained herein. If the construction, modification, or operation of a Class IV-A Incinerator by the owner/operator would not comply with all conditions of this regulation, the owner/operator must apply for and obtain an individual state air emission license before beginning the actual construction, modification, or operation of the air emissions source.

- Applicability

- This regulation applies statewide.

- This regulation supersedes Rules Concerning the Processing of Applications and Other Administrative Matters, 06-096 CMR 2, where applicable.

- This regulation applies to all Class IV-A Veterinary Incinerators (VI) and Human Crematory Incinerators (HCI) unless the unit would be required to be covered under a separate state air emission license.

- This regulation applies to a facility which is a minor source of air emissions pursuant to Department regulation 06-096 CMR 100 Definitions; that is not defined as a Part 70 source and is not subject to 06-096 CMR Emission Statements. This regulation does not exempt a minor source from any applicable state or federal requirements including 06-096 CMR 143 New Source Performance Standards or 06-096 CMR 144 National Emission Standards for Hazardous Air Pollutants.

- This regulation applies if the incinerator unit has a maximum heat input less than 10.0 MMBtu/hr.

- This regulation applies if the aggregate of all fuel burning equipment at a facility, under control of the owner/operator burns less than 65,000 gallons of diesel, #2, #4, or #6 fuel oil or equivalent natural gas/propane fuel in a calendar year.

- Exclusions. A source shall not be issued a GP if:

(1)	It is determined by the Department that the source cannot comply with the terms and conditions of this regulation;

(2)	The Department has reasonable cause to believe that the application contains fraud or misrepresentation; or

(3)	The person applying for the General Permit failed to disclose a material fact required by the application or the regulations of which the applicant had, or should have had, knowledge at the time the application was submitted.

(4)	The owner/operator owes past due fees or civil penalties to the Department from previous licenses, permits or consent agreements.

- Definitions

The following terms, as used in this Chapter, have the following meanings:

- Authorized Official. “Authorized Official” means any duly authorized person given permission by an owner/operator to conduct business with the Department on their behalf.

- Facility. “Facility” means the aggregate of all the non-temporary pollutant-emitting activities which are located on one or more contiguous or adjacent properties and are under the control of the same person (or persons under common control) as defined in 06-096 CMR 100 Definitions.

- Class IV-A Human Crematory Incinerator. “Class IV-A Human Crematory Incinerator” means crematory and pathological incinerators, suitable for human remains such as bodies, organs, and solid organic wastes from hospitals, laboratories and similar sources consisting of up to 85% moisture, 5% incombustible solids, and a heating value of approximately 1000 BTU/lb as fired.

- Class IV-A Veterinary Incinerator. “Class IV-A Veterinary Incinerator” means any crematory or pathological incinerators, suitable for the cremation of animal remains such as carcasses, organs, and solid organic wastes consisting of up to 85% moisture, 5% incombustible solids, and a heating value of approximately 1000 BTU/lb as fired.

- Incinerator Operator. “Incinerator Operator” means any person designated to operate a Veterinary Incinerator or a Human Crematory Incinerator unit in accordance with the manufacturers operating guidelines under the supervision of an Owner/Operator.

- Owner/Operator. “Owner/Operator” means any person who owns or has direct control or supervision over a regulated Class IV-A Veterinary Incinerator or Human Crematory Incinerator, and who receives a Class IV-A Incinerator General Permit under the terms of this regulation.

- Safe Access. “Safe Access” means reasonable access to the regulated facility which complies with safety requirements of any local, state, or federal regulating authority.

- Terms and Conditions for Processing Class IV-A Incinerator General Permit Applications

- Registration of Owner. Prior to the construction, modification or operation of a Class IV-A Veterinary Incinerator or Human Cremator Incinerator, the owner shall either obtain an air emission license pursuant to the requirements of 06-096 CMR 115 Major and Minor Source Air Emission License Regulations, or apply to register the equipment with the Department to obtain a General Permit (GP) for each unit to be operated. The owner/operator must submit a Notice of Intent to Comply (NOITC) to the Department as part of the general permit application. The Department will then issue a GP notification letter that will notify the applicant when they can commence operation.

- Required GP Application Form and Additional Information. The application for a general permit shall include an application form prescribed by the Department and any other additional information required by the Department, unless otherwise specified by this Chapter. The application may not omit information needed to determine the applicability of this rule. The application form and the additional required information shall include, but is not limited to, the following elements:

  - Identifying information, including contact information for the owner/operator;

  - The manufacturer, model, date(s) of manufacture and installation, and maximum charging rate of the incinerator to be issued a GP;

  - A unique identifier, such as a serial number, etc. associated with this GP;

  - Any other information that may be necessary to implement and enforce any state or federal air emission control requirements applicable to the source;

  - If required by the Department, proposed monitoring, testing, record keeping and reporting protocols, and results of previously performed performance tests;

  - A certification statement as set forth in this Section; and

  - A copy of the public notice newspaper tear sheet.

- Equipment Identification Label. Once an application has been received, the Department will assign a unique identifier general permit number (GPN) for each Class IV-A Veterinary Incinerator or Human Crematory Incinerator. The Class IV-A Incinerator associated with the GPN shall be clearly marked (engraved, stenciled, etched, or otherwise permanently affixed) with one of the following:

  - The current GPN number, or

  - A serial number or other unique equipment number that is also listed in the GPN application and which can easily be cross referenced.

- Notice of Intent to Comply. A Notice of Intent to Comply (NOITC) certification statement is required as part of the standard General Permit application. A copy of the NOITC shall also be sent to the municipality where the equipment will be located, except in the case of an unorganized territory where notification will be made to the respective county commissioners. Once the General Permit is issued, the owner/operator is bound by the conditions of this regulation and must comply with any and all applicable conditions until such time as the owner/operator informs the Department, in writing, that they no longer intend to operate the listed equipment or the owner/operator applies for and obtains a Chapter 115 air emission license.

- Required NOITC Information. The NOITC shall be a section of the application form that is prescribed by the Department. The NOITC information shall include, but is not limited to, the following elements:

  - A previously assigned GPN or manufacturers serial number that is permanently marked on the unit;

  - Identifying information, including location and contact information for the owner/operator;

  - A statement that the owner/operator intends to comply with and operate the listed equipment to the terms and conditions set forth in this chapter;

  - Any other information that may be necessary to implement and enforce any federal or state air emissions control requirements applicable to the source; and

  - A compliance certification statement as set forth in this Section.

- Certification Statement. All General Permit applications submitted to the Department in accordance with this chapter shall contain an NOITC certification of truth, accuracy, and completeness with the signature and printed name of either the owner/operator or responsible official as defined in Section 2 of this Chapter. Pursuant to 06-096 CMR 100 Definitions, signatures of authorized officials must be accompanied by a signed statement from the owner/operator or responsible official, giving them the authority to sign on their behalf. The signatory sheet shall make the following certification:

"I certify that the equipment listed in this application shall be operated in compliance with the terms and conditions of 060-096 CMR 165 General Permit for Class IV-A Incinerators and any other state or federal air emission control regulations that are applicable. I certify under penalty of law that I have personally examined the information submitted in the document and all attachments thereto and that, based on my inquiry of those individuals immediately responsible for obtaining the information, I believe the information is true, accurate, and complete. I authorize the Department to enter the property that is the subject of this application, at reasonable hours, including buildings, structures or conveyances on the property, to determine the accuracy of any information provided herein. I am aware there are significant civil and criminal penalties for submitting false information, including the possibility of fine and imprisonment. I certify that this operation shall fully comply with all General Permit regulations for this type of facility/unit.”

In the event of becoming aware that incorrect information was submitted, the responsible/authorized official must provide the Department with the supplementary facts or corrected information.

G.	Public Notice of Intent to File. A public notice of intent to file under Chapter 165 shall be published in a local newspaper no more than 30 days before an application is submitted. A copy of the tear sheet is required for the processing of a General Permit application.

H.	Fees. The owner/operator shall pay an annual fee to the Department pursuant to 38 MRSA §353-A(4). The first year’s fee is due with the GPN application form. Payment of the annual air emission permit fee is required for continuous activation of the General Permit. A general permit shall be deactivated if the permit fee is not paid with 60 days of the annual fee due date shown on the invoice. If a permit is deactivated, the owner/operator must reapply for a general permit when they want to resume operations.

I.	Application Submittal. An application for a General Permit and NOITC must be filed with the Department of Environmental Protection. Applications may be submitted via fax provided the original application is received by the Department within seven calendar days.

- Source obligation. Neither a General Permit nor an NOITC shall relieve any owner/operator of a source from the responsibility to fully comply with any other requirements applicable to the source.

- Public access to information and confidentiality. All information and data submitted to the Department shall be subject to the provisions of the Freedom of Access Law, Title 1 MRSA §401 et seq., as amended. Documents which the applicant believes may not be subject to disclosure under the Freedom of Access Law should be clearly marked as “claimed confidential” at the time of submission. Such a claim of confidentiality does not itself protect the documents from disclosure, but alerts the Department to the applicant’s position that the documents may not be subject to disclosure. Public records include, but are not limited to, the following:

- Information concerning the nature and extent of the emissions of any regulated pollutant by a source; and

- Information submitted by the source with respect to the economic, environmental and energy impacts of various control options in the determination of the control technology requirements.

- Class IV-A Incinerator General Permit Conditions and Emission Control Requirements

- General Conditions. Notwithstanding any part of this regulation, the owner/operator is subject to the applicable parts of 06-096 CMR 101 Visible Emissions.

- Employees and authorized representatives of the Department shall be allowed safe access to the owner/operator’s business premises during business hours, or any time during which any emissions units are in operation, and at such other times as the Department deems necessary for the purpose of performing tests, collecting samples, conducting inspections, or examining and copying records relating to emissions pursuant to Title 38 MRSA §347-C. Prior to entrance on the premises, the Department shall notify the owner/operator, or a designee of a need to inspect the facility. The owner/operator shall provide safe access that complies with safety requirements of any local, state, or federal regulating authority.

- The General Permit does not convey any property rights of any sort, or any exclusive privilege.

- The owner/operator shall maintain sufficient operational records to accurately document compliance with emission standards and general permit conditions as stipulated in this Chapter and shall maintain such records for a minimum of six (6) years. The records maintained on site shall also include a copy of the GP Application, applicable NOITC forms and the manufacturers operating instructions. Records shall be submitted to the Department upon written request.

- The owner/operator shall comply with all applicable terms and conditions of this general permit. The filing of an appeal, the notification of planned changes or anticipated noncompliance, or the filing of an application for an air emissions license pursuant to 06-096 CMR 115 shall not override any condition of this general permit.

- The owner/operator shall not use as a defense in an enforcement action that the disruption, cessation, or reduction of operations would have been necessary in order to maintain compliance with the conditions of the general permit.

- Notwithstanding any other provisions in the State Implementation Plan approved by the EPA or Section 114(a) of the CAA, any credible evidence may be used for the purpose of establishing whether a person has violated or is in violation of any statute, regulation, or Part 70 license requirement.

- Upon written request from the Department, the owner shall establish and maintain such records, make such reports, install, use and maintain such monitoring equipment, sample such emissions (in accordance with such methods, at such locations, at such intervals, and in such a manner as the Department shall prescribe), and provide other information as the Department may reasonably require to determine compliance status.

- The owner/operator shall keep a copy of the NOITC form available at the site where the facility operates and provide it to Department employees upon request.

- The owner/operator shall insure that all persons authorized as incinerator operator(s) be familiar with the terms and conditions of this general permit.

- Class IV-A Incinerator Operation and Emission Control Requirements. The owner/operator shall maintain and operate the emission control equipment on the Class IV-A Veterinary Incinerator or Human Crematory Incinerator as necessary to maintain temperatures and limit visible emissions to the opacity standard listed in this Subsection and the manufacturers operating guidelines.

(1)	The owner/operator shall establish and maintain a continuing program of best management practices for suppression of fugitive particulate matter during any period of operation which may result in fugitive emissions, and shall submit a description of the program to the Department upon request.

(2)	The incinerator shall be used for the disposal of Type 4 Waste in Class IV-A Veterinary Incinerators or Human Crematory Incinerators, as defined in 06-096 CMR 100 Definitions Regulation. Class IV-A Incinerators shall not be used for the disposal of plastics, cytotoxic (antineoplastic) drugs or any radioactive wastes and shall not be used to dispose of any medical waste classified as Type 7 Infectious waste, as defined in 06-096 CMR 100 Definitions. However, the incidental use of plastics used in wrapping animal carcasses for handling and storage purposes is allowed.

(3)	The incinerator shall not exceed its maximum design charging rate. Auxiliary fuel input to the primary and secondary chamber shall be Liquid Propane (LP), Liquid Natural Gas (LNG), Natural Gas (NG) or #2 fuel oil.

(4)	The incinerator combustion gases shall vent to a stack which has a stack height of at least 60 percent of Good Engineering Practice (GEP), based upon the facility building dimensions.

(5)	An operational log shall be maintained recording the weight of each charge to the incinerator, preheat temperature, preheating time, charging time, afterburner temperature directly after charging and every 60 minutes after startup until and including final shutdown time. The facility may make use of a temperature chart recorder on the incinerator unit to record the start time, date, and the weight of remains charged.

    - The secondary chamber of the incinerator shall be maintained at or above 16000 F prior to commencing the burn cycle and shall be maintained at or above 16000 F throughout the duration of the burn.

    - Once the burn cycle has commenced by introduction of primary chamber combustion, the incinerator shall be operated in an efficient manner and as specified by the manufacturer for the period of time between preheat and reaching the set operational temperature to be a minimum of 1600° F in the secondary chamber.

    - A pyrometer and ¼ inch test port shall be maintained at that location of the incinerator or refractory lined stack which provides sufficient volume to insure a flue gas retention time of not less than 1.0 seconds at minimum temperature of 16000 F.

    - The owner/operator shall not exceed a particulate matter emission limit of 0.12 gr/dscf (grains per dry standard cubic foot) corrected to 12 percent CO2 from the auxiliary fuel.

    - Visible emissions from the incinerator shall not exceed an opacity limit of 10 percent based on a six (6) minute block average basis.

    - All persons designated as incinerator operator(s) shall receive adequate training to operate the incinerator in accordance with the manufacturer’s specifications, and shall be familiar with the terms of the General Permit as it pertains to the operation of the incinerator.

- General Permit Class IV-A Incinerator Record Keeping, Testing and Reporting Requirements

A.	Record Keeping Requirements

(1)	The owner/operator shall maintain sufficient records to accurately document compliance with emission standards and general permit conditions and shall maintain such records for a minimum of six (6) years. The records shall be submitted to the Department upon written request.

      - The owner/operator shall maintain a log detailing and quantifying the hours of operation on a daily basis for each Class IV-A Veterinary Incinerator or Human Crematory Incinerator. The log shall record the weight of each charge to the incinerator, preheat temperature, preheating time, charging time, afterburner temperature directly after charging and every 60 minutes after startup until and including final shutdown time. The operation log shall be kept on-site at the incinerator location.

      - The owner/operator shall maintain a log detailing the maintenance of emission control equipment. Records of the date of each inspection and any corrective action required will be included in the maintenance log. The maintenance log shall be kept on-site at the incinerator location.

      - The owner/operator shall maintain records of malfunctions, failures, downtime, and any other similar change in operation of air pollution control systems or the emissions unit itself that would affect emissions and that is not consistent with the terms and conditions of this general permit.

      - Although it is not being required at this time, the installation and operation of continuous recording devices may become necessary to document compliance with the temperature requirements of this permit. If the Department determines that continuous recording devices are necessary, the permit holder shall, within 120 days, demonstrate that continuous recorders have been installed and are operational and recording accurately.

B.	Testing and Reporting Requirements

(1)	In accordance with the Department’s air emission compliance test protocol and 40 CFR Part 60 or other approved method as required by the Department, the owner/operator shall:

(a)	perform testing to demonstrate compliance with applicable emission standards under circumstances representative of the facility’s normal process and operating conditions. Testing shall be completed within sixty (60) calendar days of receipt of a notification to test from the Department or EPA, if visible emissions, equipment operating parameters, staff inspection, air monitoring or other cause indicate that equipment may be operating out of compliance with emission standards or license conditions;

(b)	install or make provisions to install test ports that meet the criteria of 40 CFR Part 60, Appendix A, and test platforms, if necessary, and other accommodations necessary to allow emission testing; and

(c)	submit a written report of test results to the Department within thirty (30) days from date of test completion.

(3)	If the results of a test performed under circumstances representative of the facility’s normal process and operating conditions indicate emissions in excess of the applicable standards, then:

(a)	within thirty (30) days following receipt of such test results, the owner/operator shall correct malfunctions in the system and shall re-test the non-complying emission source under circumstances representative of the facility’s normal process and operating conditions and in accordance with the Department’s air emission compliance test protocol and 40 CFR Part 60 or other method approved or required by the Department;

(b)	the days of violation shall be presumed to include the date of test and each and every day of operation thereafter until compliance is demonstrated under normal and representative process and operating conditions, except to the extent that the facility can prove to the satisfaction of the Department that there were intervening days during which no violation occurred or that the violation was not continuing in nature; and

(c)	the owner/operator may, upon the approval of the Department following the successful demonstration of compliance at alternative load conditions, operate under such alternative load conditions on an interim basis prior to a demonstration of compliance under normal and representative process and operating conditions.

(4)	The owner/operator shall notify the Department within two (2) days or the next state working day, whichever is later, of occasions where malfunctions, failures, downtime, and any other similar change in operation of air pollution control systems or the emissions unit itself that result in an increase of emissions. The owner/operator shall report all excess emissions in the units of the applicable emission limitation.

(5)	Upon written request from the Department, the owner/operator shall establish and maintain records, make reports, install, use and maintain monitoring equipment, and sample emissions in accordance with prescribed methods, at locations, intervals, and in a manner the Department shall prescribe; and provide other information as the Department may reasonably require to determine compliance status.

C.	Severability. Each part of this Chapter is severable, and in the event that any part of this Chapter is held to be invalid, the remainder of the Chapter continues in full force and effect.

___________________________________________

AUTHORITY: 38 M.R.S.A., §585-A

EFFECTIVE DATE: May 6, 2012 – filing 2012-136