Chapter 104:	INCINERATOR PARTICULATE EMISSION STANDARD

SUMMARY:  This regulation establishes a limitation on the amount of particulate matter allowed to be emitted from each of several catagories and sizes of incinerators and a limitation on the opacity of emissions from all incinerators.

1.	Scope.  This regulation shall be applicable to all incinerators and shall be effective in all regions in the State of Maine as follows:

A.	Immediately for all new incinerators, the construction or operation of which begins after the effective date of this emission standard unless such incinerators are required to meet a stricter standard of performance;

B.	June 1, 1975 for all existing incinerators.

2.	Emission Standard.  No person shall emit or cause to be emitted any particulate air contaminants from:

A.	Any incinerator darker than a number 1 on the Ringelmann Chart, excluding the emission of water vapor;

B.	Any Class III, IV, V, VI, and VII incinerator having a designed charging rate of 50 tons per day or less, that exceed 0.2 grains per standard cubic foot of dry flue gas for a two hour sampling period corrected to 12 percent carbon dioxide without the contribution of carbon dioxide from the auxiliary fuel.

C.	Any class incinerator having a designed charging rate greater than 50 tons per day, that exceed 0.08 grains per standard cubic foot of dry flue gas for a 2-hour sampling period, corrected to 12 percent carbon dioxide without the contribution of carbon dioxide from the auxiliary fuel.

3.	Incinerator Malfunction or Breakdown.  Any person owning or operating incinerators under the scope of this emission standard, except Class I, IA, II, and IIA incinerators that malfunction or suffer a breakdown in any component part which malfunction or breakdown causes this emission standard to be violated shall notify the Department within 48 hours in writing.

4.	Test Methods and Procedures.  Test Methods 1, 3, and 5 as promulgated by the Administrator, Environmental Protection Agency Rules and Regulations 60.85, "Federal Register", Volume 36, Number 247, Thursday, December 23, 1971 or such other methods as are deemed equivalent by the Department shall be those used to determine compliance with this regulation.

AUTHORITY:	38 M.R.S.A., Section 585, 585-A

EFFECTIVE DATE:	January 31, 1972

Amended:  October 15, 1979

Amended:  November 3, 1990

EFFECTIVE DATE (ELECTRONIC CONVERSION):	May 8, 1996

_______________________________________________

BASIS STATEMENT

The basis of the regulation is to limit particulate emissions to protect the ambient air quality standard.

BASIS STATEMENT FOR AMENDMENT OF SEPTEMBER 26, 1990

During the 114th Session of the Maine Legislature, the Board of Environmental Protection was given the authority under 38 M.R.S.A. 585 and 585-A to establish and amend emission standards and regulations to implement ambient air quality standards and emission standards.  This rule was amended to reflect the new authority under statute.  No request for public hearing nor comments were received on the reauthorization.