Chapter 105:	GENERAL PROCESS SOURCE PARTICULATE EMISSION STANDARD

SUMMARY:  This regulation establishes a limitation on the amount of particulate emissions allowed from any general process source determined on the basis of the size and rate at which the process operates.  Also included are specific emission limitations on Kraft pulping processes based on the tons of pulp produced.

1.	Scope.  A general process source shall be any emission source except fuel-burning equipment, incinerators, mobile sources, open burning sources, and sources of fugitive dust.  This emission standard shall be effective in all regions in the State of Maine as follows:

A.	Immediately for all new process sources the construction of which begins after the Commission adopts this emission standard unless such sources are required to meet a stricter standard of performance;

B.	June 1, 1975 for all existing sources.

2.	Emission Standard.  Any person operating any general process source within the scope of this regulation, except kraft pulping processes, shall limit the emission of particulate air contaminants from such emission source in accordance with Section 3, for a 2-hour sampling period.

Any person operating any kraft pulping process shall limit the emission of particulate air contaminants from such emission source as follows:  4 pounds of particulate emissions per air dried ton of kraft pulp from the recovery boiler, 1 pound of particulate air contaminants per air dried ton of kraft pulp from the lime kiln, 0.5 pound of particulate air contaminants per air dried ton of kraft pulp from the smelt tank for a 2-hour sampling period.

All similar units, processes operated by the same person at the same general location shall be combined in computing the process weight rate for use in Table 105A.

3.    Table 105A

Process Weight Rate	Emission Rate

(lb./hr.)	(lbs./hr.)

50	0.36

100	0.55

500	1.53

1,000	2.25

5,000	6.34

10,000	9.73

20,000	14.99

60,000	29.60

80,000	31.19

120,000	33.28

160,000	34.85

200,000	36.11

400,000	40.35

1,000,000	46.72

Interpolation of Table 105A for process weight rates up to 60,000 lbs./hr. shall be computed by use of the following equation:

E = 3.59P0.62for P less than or equal to 30 tons/hr.

and interpolation and extrapolation of Table 105A for rates in excess of 60,000 lbs./hr. shall be computed by use of the equation:

E = 17.31P0.16for P greater than 30 tons/hr.

Where E = Emissions in pounds per hour and P = process weight rate in tons per hour.

4.	Malfunction or breakdown.  Any person owning or operating a source under the scope of this emission standard that malfunctions or suffers a breakdown in any component part that causes this emission standard to be violated shall notify the Commission within 48 hours in writing.

5.	Test Methods and Procedures.  Test methods 1 and 5 as promulgated by the Administrator, Environmental Protection Agency Rules and Regulations 60.85, "Federal Register", volume 36, number 247, Thursday, December 23, 1971 or such other methods as are deemed equivalent by the Environmental Improvement Commission shall be used to determine compliance with this regulation.

AUTHORITY:	38 M.R.S.A., Section 585, 585-A

EFFECTIVE DATE:	January 31, 1972

Amended:  November 3, 1990

EFFECTIVE DATE (ELECTRONIC CONVERSION):	May 8, 1996

BASIS STATEMENT

The basis of the regulation is to limit particulate emissions from this catagory of sources so as to protect the ambient air quality standard.

BASIS STATEMENT FOR AMENDMENT OF SEPTEMBER 26, 1990

During the 114th Session of the Maine Legislature, the Board of Environmental Protection was given the authority under 38 M.R.S.A. 585 and 585-A to establish and amend emission standards and regulations to implement ambient air quality standards and emission standards.  This rule was amended to reflect the new authority under statute.  No request for public hearing nor comments were received on the reauthorization.