Chapter 586:	RULES PERTAINING TO DISCHARGES TO CLASS A WATERS

SUMMARY: This rule establishes criteria to define what constitutes effluent quality necessary to ensure the standards for class A waters are met.

1.	Scope. Under 38 MRSA section 464 discharges to class A waters must be equal to or better than the receiving water in order to ensure that habitat, aquatic life, and bacteria are as naturally occurs. The following sections define effluent criteria necessary to ensure these requirements are met.

2.	Criterion for pH. The pH of the discharged effluent shall not be greater than or less than a 0.2 pH unit difference from that of the seasonal median value of the receiving water upstream of the discharge.

3.	Criterion for plant nutrients. Nutrients in the discharged effluent shall not exceed the seasonal median concentration of nutrients in the receiving water, or a value demonstrated by the applicant to be better than the seasonal median and which does not cause the aquatic life to be other than as naturally occurs.

The effluent shall not significantly alter the particle size distribution of the downstream floral community or otherwise alter the natural character of the downstream biotic community.

4.	Criterion for temperature. The temperature of the discharged effluent shall not vary by more than 0.5°F from the temperature of the receiving water at the time of discharge.

5.	Criterion for dissolved oxygen. In addition to the requirements of 38 MRSA section 465(2)(B) the dissolved oxygen content of the discharged effluent shall not be less than that of the receiving water at the time of discharge.

6.	Criteria for other water quality parameters. Except as provided above, the concentration in the discharged effluent of biochemical oxygen demand and all constituents listed in Quality Criteria for Water 1986 (EPA 440/5-86-001) shall not exceed the seasonal median concentration as measured in the receiving water upstream of the discharge or prior to a discharge where a suitable upstream site is not available.

7.	Establishment of seasonal values. For the purpose of establishing seasonal values in the receiving water pursuant to Sections 2, 3, and 6 of this rule, an applicant will provide data based on seasons and sample frequencies approved by the Department on a case by case basis.

________________________________________________

AUTHORITY: 38 M.R.S.A. Section 343-A, 464(5), and 465(2)

EFFECTIVE DATE: February 18, 1989

Accepted for filing: February 13, 1989

EFFECTIVE DATE (ELECTRONIC CONVERSION): May 4, 1996

MINOR CORRECTION (HEADER): June 10, 2015